# fFlight simulator V3.x

## PC preparation for developer

Install:

- python version 3.11.9

- qt creator 18.0.0 community Edition

- install PyCharm 2025.2.4

## For user and developers

### x-plane

- installation from X-Plane | The world’s most advanced flight simulator.

check if streaming is running on x-plane (assuming x-plane is running on the same pc) Adapt IP address, if x-plane is NOT on same computer

MSFS2024

- Install MSFS2024 (via steam)

- Start -> Settings -> General -> Advanced options -> enable developer mode

- On top screen menu insall SDK

You should have than :

- Install Mobiflight https://www.mobiflight.com/, this enables Telemetry handling
-> while installation chose WASM module for MSFS2024 as well

- Check in MSFS2024 if WASM is enabled

MotionCompensation

- Install openXR MotionCompensation Releases · BuzzteeBear/OpenXR-MotionCompensation

- Adapt settings:

[startup]

physical_enabled = 0

…

[tracker]

type = yaw

side = none

role = none

…

Add at the end of the file

[motion_source]

source_type = mmf

mmf_name = OxrMcInput

mmf_format = euler_deg

🏗️ High-Level Architecture

# SimMotion Software

## MSFSListener

Thread type: QThread Purpose: Fetch raw telemetry from MSFS at high frequency Tech: SimConnect + MobiFlightVariableRequests

Outputs (via Qt signal):

- pitch (deg)

- roll (deg)

- yaw (deg true)

- airspeed (knots)

- frame_rate (Hz)

- timestamp

Key features:

- Auto-reconnect

- Graceful fallback (zeros when telemetry missing)

- High-frequency polling (50 Hz)

- No scaling, no filtering → pure raw data

## MotionPlatformThread

Thread type: QThread Purpose: Convert raw telemetry into motion commands and send to Teensy

Inputs:

- Raw telemetry from MSFSListener

- Scaling factors from GUI (SimMotion)

- Motion enable/disable toggle

Responsibilities:

- Compute Euler rates (pitch_rate, roll_rate, yaw_rate)

- Apply scaling factors

- Apply deminute curves based on airspeed

- Format Teensy packet:

X;pitch_angle;pitch_rate;roll_angle;roll_rate;yaw_angle;yaw_rate;airspeed\n

- Send serial data to Teensy

- Emit outgoing telemetry for GUI

## WitMotionSensor

Thread type: Worker thread Purpose: Read IMU data and provide rig orientation

Outputs:

- roll_adj

- pitch_adj

- yaw_adj

Responsibilities:

- Read IMU via serial

- Apply calibration offsets

- Write orientation to MMF (for VR motion compensation)

- Update GUI labels asynchronously

## FlightStick

Purpose: Hardware toggle for motion enable/disable

Responsibilities:

Detect button press (button 7)

Emit Qt signal to toggle motion platform

## SimMotion (Main GUI Application)

Purpose: Orchestrates all components Responsibilities:

- Start/stop all threads

- Manage stop_event for clean shutdown

- Provide scaling factors to MotionPlatformThread

- Display incoming/outgoing telemetry

- Display IMU orientation

- Provide user controls (connect, motion toggle, quit)

- Handle errors and status messages

## Data Flow Summary

### Telemetry Path

MSFS → SimConnect → MobiFlight → MSFSListener → Qt Signal → MotionPlatformThread → Teensy

### IMU Path

WitMotion IMU → Serial → WitMotionSensor → MMF + GUI

### Motion Toggle Path

FlightStick → Qt Signal → SimMotion → MotionPlatformThread

### GUI Update Path

MSFSListener → SimMotion GUI MotionPlatformThread → SimMotion GUI WitMotionSensor → SimMotion GUI

## Threading Model

| Component | Thread Type | Communication Method |
| --- | --- | --- |
| MSFSListener | QThread | Qt Signal (dict) |
| MotionPlatformThread | QThread | Qt Signal (dict) |
| WitMotionSensor | Worker | Direct callback + Qt |
| FlightStick | Internal | Qt Signal |
| GUI (SimMotion) | Main Thread | Qt event loop |

## Separation of Concerns

| Layer | Responsibility |
| --- | --- |
| SimMotion | Orchestration, GUI, configuration |
| MSFSListener | Raw telemetry acquisition |
| MotionPlatformThread | Motion cueing logic + Teensy output |
| WitMotionSensor | Rig orientation + MMF |
| FlightStick | Hardware toggle |

Installation

Install x-plane 12 or MSFS2024

Install simHub SimHub | Pro Sim Racing

# HW

# Firmware

Protocol concept

X;timestamp;pitch;pitch_rate;roll;roll_rate;yaw;yaw_rate;air_speed

Field; Purpose. X; Message type or header (x plane closed loop). timestamp; Optional for Teensy unless you need sync or logging. pitch; Degrees (float), aircraft nose up/down. pitch_rate; Degrees/sec (float), angular velocity. roll; Degrees (float), aircraft bank. roll_rate; Degrees/sec (float), angular velocity. yaw; Degrees (float), heading. yaw_rate; Degrees/sec (float), angular velocity. air_speed; Knots or m/s (float), for dynamic gain adjustment.

Example

X;ts=1698945600.123;mode=POS;pitch=12.5;pitch_rate=3.2;roll=-5.1;roll_rate=-1.8;yaw=90.0;yaw_rate=0.0;airspeed=120.3

Available commands:

X;... -> live motion stream from X-Plane

H; -> homes pitch and roll to center

E; -> prints current pitch, roll, yaw positions

P;[angle] -> move pitch to [angle] (±30° limit)

R;[angle] -> move roll to [angle] (±30° limit)

Y;[angle] -> move yaw to [angle] (step count only)

?; -> prints this help menu

# Simulator preparations

## x-plane preparation

|  | 12 Btn 9: Toggle enabling of VR hardware 11 Btn 8: Hold brake left hack to reload motion compensation of VR 7 Btn 4: Reset VR View _______________________________________________ 10 Btn 7: Reset flight to most recent start 9 Btn 6: Toggle brakes max effort hack to set motion platform on and off 8 Btn 5: Toggle simulation paused state _______________________________________________ 15 Do nothing |
| --- | --- |

Bild – usb controller

10 – 8

9 – 7 toggle platform

8 – 6

12-10

11-9 toggle motion compensation

7-5

15-13

# OpenXR Motion compensation

9.11.2025

Download OpenXR-MotionCompensation from

Releases · BuzzteeBear/OpenXR-MotionCompensation

Make a copy of

"C:\Users\[ YOUR_USERNAME]\AppData\Local\OpenXR-MotionCompensation\OpenXR-MotionCompensation.ini"

To

"C:\Users\[YOUR_USERNAME]\AppData\Local\OpenXR-MotionCompensation\X-Plane.ini"

Adapt the section in X-Plane.ini

[startup] enabled = 1 auto_activate = 1 auto_activate_delay = 0 physical_enabled = 0

add this sections at the end:

[axes] compensate_roll = true compensate_pitch = false compensate_yaw = false

Restart VR link (e.g. meta quest link  Settings  Beta  Restart Meta Quest Link)

Start x-plane  start a flight  start VR mode and test motion compensation with

openxr_injection_test.py

Click Inject Yaw  this should turn you view 360° around yaw

# Send motion platform status to cockpit

Install FlyWithLua from here
FlyWithLua NG+ (Next Generation Plus) Edition for X-Plane 12 (Win, Lin, Mac) - Utilities - X-Plane.Org Forum

(mainly copy folder FlyWithLua to C:\X-Plane 12\Resources\plugins)

create a file SimMotion.lua and SimMotionStatus.txt

SimMotion.lua example

# Addon scenery

Requires

OpenSceneryX – Kostenlose Szenerieobjekte für X-Plane®

Than download scenery from store.x-plane.com

Switzerland Photorealistic XP Installation Extract all folders (total number is 53) into xplane Custom scenery folder. look for a file called: scenery.packs.ini file (located inside xplane custom scenery folder). Copy and paste the follow lines into scenery.packs.ini as they are: MAKE SURE airports or cityscapes scenery are above the Switzerland Ortho scenery including default Global Airports: See example below: SCENERY_PACK Custom Scenery/MY AIRPORT ADDON/ SCENERY_PACK *GLOBAL_AIRPORTS* SCENERY_PACK Custom Scenery/Switzerland_Ortho_Forests/ SCENERY_PACK Custom Scenery/Switzerland_Ortho_Forests_Exclude/ SCENERY_PACK Custom Scenery/switzerland_ortho_B_05/ SCENERY_PACK Custom Scenery/switzerland_ortho_B_06/ SCENERY_PACK Custom Scenery/switzerland_ortho_B_07/ SCENERY_PACK Custom Scenery/switzerland_ortho_B_08/ SCENERY_PACK Custom Scenery/switzerland_ortho_B_09/ SCENERY_PACK Custom Scenery/switzerland_ortho_B_10/ SCENERY_PACK Custom Scenery/switzerland_ortho_B_11/ SCENERY_PACK Custom Scenery/switzerland_ortho_B_12/ SCENERY_PACK Custom Scenery/switzerland_ortho_C_01/ SCENERY_PACK Custom Scenery/switzerland_ortho_C_02/ SCENERY_PACK Custom Scenery/switzerland_ortho_C_03/ SCENERY_PACK Custom Scenery/switzerland_ortho_C_04/ SCENERY_PACK Custom Scenery/switzerland_ortho_C_05/ SCENERY_PACK Custom Scenery/switzerland_ortho_C_06/ SCENERY_PACK Custom Scenery/switzerland_ortho_C_07/ SCENERY_PACK Custom Scenery/switzerland_ortho_C_08/ SCENERY_PACK Custom Scenery/switzerland_ortho_C_09/ SCENERY_PACK Custom Scenery/switzerland_ortho_C_10/ SCENERY_PACK Custom Scenery/switzerland_ortho_C_11/ SCENERY_PACK Custom Scenery/switzerland_ortho_C_12/ SCENERY_PACK Custom Scenery/switzerland_ortho_B_02/ SCENERY_PACK Custom Scenery/switzerland_ortho_B_03/ SCENERY_PACK Custom Scenery/switzerland_ortho_B_04/ SCENERY_PACK Custom Scenery/switzerland_ortho_A_07/ SCENERY_PACK Custom Scenery/switzerland_ortho_A_08/ SCENERY_PACK Custom Scenery/switzerland_ortho_A_09/ SCENERY_PACK Custom Scenery/switzerland_ortho_A_10/ SCENERY_PACK Custom Scenery/switzerland_ortho_A_05/ SCENERY_PACK Custom Scenery/switzerland_ortho_A_06/ SCENERY_PACK Custom Scenery/switzerland_ortho_A_04/ SCENERY_PACK Custom Scenery/switzerland_ortho_A_03/ SCENERY_PACK Custom Scenery/switzerland_ortho_D_02/ SCENERY_PACK Custom Scenery/switzerland_ortho_D_03/ SCENERY_PACK Custom Scenery/switzerland_ortho_D_04/ SCENERY_PACK Custom Scenery/switzerland_ortho_D_05/ SCENERY_PACK Custom Scenery/switzerland_ortho_D_06/ SCENERY_PACK Custom Scenery/switzerland_ortho_D_07/ SCENERY_PACK Custom Scenery/switzerland_ortho_D_08/ SCENERY_PACK Custom Scenery/switzerland_ortho_D_09/ SCENERY_PACK Custom Scenery/switzerland_ortho_D_10/ SCENERY_PACK Custom Scenery/switzerland_ortho_D_11/ SCENERY_PACK Custom Scenery/switzerland_ortho_D_12/ SCENERY_PACK Custom Scenery/switzerland_ortho_E_01/ SCENERY_PACK Custom Scenery/switzerland_ortho_E_03/ SCENERY_PACK Custom Scenery/switzerland_ortho_E_04/ SCENERY_PACK Custom Scenery/switzerland_ortho_E_05/ SCENERY_PACK Custom Scenery/switzerland_ortho_E_06/ SCENERY_PACK Custom Scenery/switzerland_ortho_E_07/ SCENERY_PACK Custom Scenery/switzerland_ortho_E_08/ SCENERY_PACK Custom Scenery/switzerland_ortho_E_09/ SCENERY_PACK Custom Scenery/switzerland_ortho_D_01/ end of install

New PC

Windows

Windows 11 Pro | Für professionelle Nutzer | SwissSoftware24

PC

Captiva R89-504 - kaufen bei Galaxus

VR optimisation

VR & FPS {SOLVED] - VR in X-Plane 12 - X-Plane.Org Forum

| Original |  |
| --- | --- |